Welding fume extraction questionnaire.

- Name of the end-user:	 Marketex Vision OU

City:	Tallinn, Kopli 103, Estonia
Workshop size:  60m (Length) * 37m (Width) * 22m (Height)

The purpose of the system for purification of welding gases is a particle content of not more than 2 mg/m3, the air movement of the system is not more than 2 m/s.

- What products are manufactured: Metal Constructions

Qty of welders   Amperage (А)    Hours a day

Semi-automatic welding(MIG/MAG)	  	  10                     	320А		16 h

Fully automatic welding	    		    2	              800А		3 h

- Type of consumable:	solid/flux wires

- Total use of consumable per month: 3000 kg

- What is the base material to be welded: Black steel 355

- Is this material free of oil? :  No

- Is this material free of paint?:   Primered

- Measurements of details: up to 20 x 20 mtr

- Is possible to use extraction arms ?: Yes

- What is the actual arc time per welder?: 30%

- Do we have to / can we propose Diluter System(s): Yes

- There is a crane running at 20 mtr height.

- There is a free way to install Push Pull ducting at 6/9/12 mtr height:  6m/9m/12m

- Customer has 220 / 400V (1ph / 3ph) available:  Yes

- Customer has a clean and dry compressed air available:  9 Bar

- Working hours are from 07 till 23 hour, 5 days a week.

- Supply of fresh air (from outside) is up to 45 000m3 per hour

- Actual project  building project : Yes

- Date of realization is estimated to be 02/2020